NAME: KRISHANU SOM                 ROLL NO.: 25BEC034

OBJECTIVE: RTL to GDSII implementation of Parameterized Digital Time Controller

Directory Created: DTC1

Inside DTC1, rtl directory, the following RTL source code is present

`timescale 1ns/1ps

module digital_time_controller #(

parameter N = 8

)(

input  wire             clk,

input  wire             rst_n,

input  wire             enable,

input  wire             load,

input  wire [N-1:0]     load_value,

output reg  [N-1:0]     count,

output reg              timeout

);

always @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

count   <= {N{1'b0}};

timeout <= 1'b0;

end else begin

timeout <= 1'b0; // default

if (load) begin

count <= load_value;

end

else if (enable) begin

if (count != 0) begin

count <= count - 1'b1;

if (count == 1)

timeout <= 1'b1; // asserted when reaching zero

end

end

end

end

endmodule

Inside rtl simulation directory, source code long with testbench is there

`timescale 1ns/1ps

module tb_digital_time_controller;

parameter N = 8;

reg             clk;

reg             rst_n;

reg             enable;

reg             load;

reg [N-1:0]     load_value;

wire [N-1:0]    count;

wire            timeout;

// DUT

digital_time_controller #(

.N(N)

) dut (

.clk(clk),

.rst_n(rst_n),

.enable(enable),

.load(load),

.load_value(load_value),

.count(count),

.timeout(timeout)

);

// Clock (10 ns period)

always #5 clk = ~clk;

initial begin

$dumpfile("dump.vcd");      // VCD file name

$dumpvars(0, tb_digital_time_controller);

// Init

clk        = 0;

rst_n      = 0;

enable     = 0;

load       = 0;

load_value = 0;

// Reset

#20 rst_n = 1;

// Load 5

#10;

load       = 1;

load_value = 8'd5;

#10;

load   = 0;

enable = 1;

// Let it count down

#100;

// Stop

enable = 0;

#20 $finish;

end

initial begin

$display("Time | load enable | load_value | count | timeout");

$monitor("%4t |  %b     %b    |    %0d     |   %0d  |   %b",

$time, load, enable, load_value, count, timeout);

end

endmodule

The vcs_command file compiles the source code and testbench together in such a format which is executable.

#vcs -full64 dtc_rtl.v -debug_access+all -lca -kdb

vcs -full64 dtc_tb.v -debug_access+all -lca -kdb

./simv Verdi

verdi -ssf novas.fsdb -nologo

The simv file runs the simulation of the code compiled by vcs_command.

Synthesis of RTL code leads to netlist formation.

In DC directory, rm_setup and ICCII directory, scripts, the template scripts given were modified according to parameterized digital time controller.

Then in ICCII directory, icc2_shell, the .tcl scripts of clock, floor planning, placement, power planning, and route were executed and saved in a library [DTC_LIB].

GUI of floor planning

GUI of placement

GUI of power planning

GUI of Clock Tree Synthesis (CTS)

GUI of routed design [GDSII]